Name ___________________________________   Period _____   Date _________________________

Circuits Lab

Purpose: 	Students will construct and differentiate between series and parallel circuits, measure the voltage and current across the circuit, and calculate the resistance using Ohm’s Law.

Task: 	Using the materials and procedures below, students will construct their own circuits.  Students will measure voltage using a voltmeter and an ammeter to measure current. Using voltage and current measurements, students will calculate resistance of each resistor in each circuit using Ohm’s Law.

Series Circuit with One Resistor.

Materials:  two 1.5 V dry cells (batteries) or Power Box, one switch, two bulbs, two bulb holders, two battery holders- if using batteries, five alligator clip wires, one multimeter (used for measuring voltage difference and current) or one voltmeter and one ammeter.

Procedure:

Part I: Series Circuit with One Resistor.

- Construct a series circuit that contains a power source (1 battery), a resistor, a switch and wires. Diagram your completed circuit showing the flow of energy using arrows.

Explain what happens when the switch is opened and why.

Series Circuit with Two Resistors

Connect the voltmeter and measure the voltage across the resistor. Remove the voltmeter and replace with the ammeter to measure the current of the circuit. Record your observations in your data table on the back.

# Part II: Series Circuit with Two Resistors

- Using one power source and two resistors, construct a series circuit. Diagram your circuit in the box to the right, measure voltage difference across each resistor and current. Record your observations in your data table on the back.

Series Circuit with Three Resistors

In the circuit with two bulbs, unscrew one of the bulbs.  Record and explain your observations.

- Calculate the resistance across the resistors using Ohm’s Law. Record your answer in the data table on the backside.

# Part III: Series Circuit with Three Resistors

Parallel Circuit

Add the third resistor to your circuit. Diagram the arrangement, measure the voltage across each resistor and current and record your observations in your data table on the backside.

# Part IV: Parallel Circuits

- Create a parallel circuit using all three resistors.  When you measure voltage and current, measure it across each strand.  Diagram the arrangement.

- In the parallel circuit, unscrew one of the bulbs.  Record and explain your observations

Data Table: Power source is set to _______ for the entire lab.

|  |  |  | Series Circuit |  |  |  | Parallel Circuit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step #2 1 battery, 1 resistor (Series) | Voltage (V) | V = |  |  |  |  | No Data |  |  |
|  | Current (A) | I = |  |  |  |  |  |  |  |
|  | Resistance () | R= |  |  |  |  |  |  |  |
| Step #3 1 battery, 2 resistors (Series) | Voltage (V) | V1 = |  |  | V2 = |  | No Data |  |  |
|  | Current (A) | I = |  |  |  |  |  |  |  |
|  | Resistance () | R1 = |  |  | R2 = |  |  |  |  |
| Step #6 1 battery, 3 resistors (Series) | Voltage (V) | V1 = |  | V2 = |  | V3 = | No Data |  |  |
|  | Current (A) | I = |  |  |  |  |  |  |  |
|  | Resistance () | R1 = |  | R2 = |  | R3 = |  |  |  |
| Step #7 1 battery, 3 resistors (Parallel) | Voltage (V) | No Data |  |  |  |  | V1 = | V2 = | V3 = |
|  | Current (A) |  |  |  |  |  | I = |  |  |
|  | Resistance () |  |  |  |  |  | R1 = | R2 = | R3 = |

Total V=

Total V=

Total V=

Post Lab Questions:

- What happened to the light bulbs in the series circuit when you removed one bulb? Why?

- What happened to the light bulbs in the parallel circuit when you removed one bulb? Why?

- What is one advantage of a parallel circuit?

- Is it better to have a string of lights, such as Christmas lights, connected in series or parallel?

- What types of circuits do you have in your home? What is the advantage of this?

- Compare and contrast (similarities and differences) the brightness of the bulbs in a series circuit verses a parallel circuit.

- Sketch the universal symbols for each:

- lightbulb 				b.  battery 			c.  open switch

d.  closed switch 			e.  resistor 			f.  conductor.