Series and Parallel Circuits Practice Problems

Series Circuit

1. Use the series circuit pictured at right to answer questions a-c.

a. What is the current in the circuit? _________

b. What is the voltage across R1? _________

c. What is the voltage across R2? _________

2. In this circuit, three resistors receive the same amount of current (4 amps) from a single source. Calculate the amount of voltage "dropped" by each resistor, as well as the amount of power dissipated by each resistor.  The voltage drop is calculated by multiplying current in the circuit by the resistance of a particular resistor: V = IR.

1Ω- _______________

2Ω- _______________

3Ω- _______________

3. Use the series circuit pictured to the right to answer questions a-e.

a. What is the total voltage across the bulbs? ___________

b. What is the total resistance of the circuit? _________

c. What is the current in the circuit? _______

d. What is the voltage drop across each light bulb?

e. Draw the path of the current on the diagram.

Parallel Circuit

Helpful Information: The voltage is equal across all components in a parallel circuit. Therefore, the voltage across R1 is equal to the voltage across R2 which is equal to the voltage across R3 which is equal to the voltage across the battery. The total current in the circuit is equal the sum of all individual branch currents.

4. Use the parallel circuit pictured right to answer questions a-c.

\a. What is the voltage across each resistor? _____________

b. What is the current in each branch? _________________

c. What is the total current provided by the battery? ______

5. Use the parallel circuit pictured right to answer questions a-c.

a. What is the voltage across each resistor? ___________

b. What is the current in each branch?______________

c. What is the total current provided by the battery? _________