PERCENT ERROR

Calculation of percent error is one way to make a distinction between an observed value and a true or literature value.  Percent error can be used to describe the accuracy of results of laboratory investigations if a generally accepted (true or literature value) is known:

[observed value   -   true/literature value]

percent error  =  		   true/literature value				x   100

Percent error is always positive.  Take the absolute value.

Practice Problems:

1.  The density of mercury is known to be 13.8 g/cm3.  Results of an 	______________________

experiment give that density to be 14.2 g/cm3.  Calculate the

percent error.

2.  The volume of a rectangular solid is known to be 556 cm3.  A student takes the following measurements for the dimensions of the rectangular solid:

8.34 cm  x  6.19 cm  x  10.42 cm

a.  What volume is calculated for the rectangular solid from 		______________________

these measurements?

b.  What is the percent error of these results?				______________________

ACCURACY and PRECISION

When taking measurements during a lab, you may need to know if your data is reliable.  There are two ways to check reliability.

Precision:	One way is to repeat the measurement several times.  A reliable result will give the same measurement time after time.  This is how close a set of measurements for a quantity are to each other, but still can be regardless of correctness.

Accuracy:	This is how close a measurement is to the correct (standard, literature) value for the quantity.

Two students massed the same sample on two different laboratory balances.  The results were as follows:

Balance A	12.11 g	   12.09 g		Balance B	12.1324 g	12.1322 g

Which balance is more precise?

If the mass of the sample is actually 12.1 g, which is more accurate?

# SIGNIFICANT DIGITS/FIGURES

Significant figures helps to determine the exactness of measurements.  The last place in a number is the inexact number – all others have been measured with certainty.  The lines below are the same length, but have different measurements.

ACTIVITY:

- Estimate the length of the coffee stirrer in centimeters.  Record here.	__________________

- Use the first ruler to measure the coffee stirrer.  Record the length.		__________________

- Use the second ruler to measure the coffee stirrer.  Record the length.	__________________

- Use the third ruler to measure the coffee stirrer.  Record the length.		__________________

Explain why ruler #3 is more accurate than ruler #1:

ONE WAY TO DETERMINE SIG FIGS (Atlantic-Pacific Rule)

This rule divides measurements into two kinds – those with a decimal point and those without.  Imagine a map of the United States and superimpose the measurement on top.  If a decimal point is present in the number, count significant digits from the Pacific side. (If decimal point is present, count from the Pacific side.)  If a decimal point is absent, count from the Atlantic side.

You should start counting with the first nonzero digit you find.  Thereafter, all digits, including zero, are significant.

ANOTHER WAY

1.  Digits other than zero are always significant.

2.  One or more final zeros used after the decimal point are always significant.

3.  Zeros between two other significant digits are always significant.

4.  Zeros used solely for spacing the decimal point are not significant.  The zeros are place holders only.

*Counting numbers (23 cockroaches, 3 cars, 5 ice cream cones)  and defined constants or conversions ( like 1 km = 1000 m) have an infinite amount of significant figures.

#### PRACTICE

Calculate the number of significant figures in the following measurements.

1.  	45.601 g       _______		4.  0.2460 mg       _______		7.  23 desks   _______

2.	0.00701 m    _______		5.  78,621.0 mL   _______  		8.  2.010 L     _______

3.         75,000 km   _______		6.  4.567  m	    _______		9.  0.002 m    _______